Huawei MA5683T

Create VLAN 100 Service-Port for an Already-Registered ONT

Two worked examples: tagged and untagged customer-side traffic

Purpose: This guide shows how to find an already-registered ONT by serial number, identify its GPON interface and ONT ID, find an unused service-port ID, and create a VLAN 100 service-port for tagged or untagged customer-side traffic.

# Workflow 0 — Find the ONT by serial number and choose the next free service-port ID

Always identify the registered ONT first. Do not assume the GPON interface, ONT ID, or service-port ID from a previous job.

## Step 1 — Find the ONT by SN

display ont info by-sn "<ONT_SERIAL_NUMBER>"

Replace <ONT_SERIAL_NUMBER> with the actual ONT serial number. Read the F/S/P (frame/slot/port) and ONT-ID from the result. For example, F/S/P 0/4/8 and ONT-ID 74 means the service-port command should use gpon 0/4/8 ont 74. If your software build does not accept this filter syntax, check the command help with display ont info ? and use the supported form.

## Step 2 — Check existing services on that ONT

display service-port port <F/S/P> ont <ONT_ID>

Review all existing service-ports and GEM ports. Do not create a duplicate service classification, especially for untagged traffic.

## Step 3 — Find the next free service-port ID

display service-port next-free-index

If this command is supported and returns an available index, use that number as the new service-port ID. If it is rejected by your OLT release, run the following and inspect the INDEX ID column:

display service-port all

Choose an index that is not already assigned. Do not rely only on the largest visible index without checking for gaps or platform-specific limits. The example IDs 1941 and 494 in the cases below are examples; replace them with the verified free index.

## Step 4 — Continue with the tagged or untagged case

Once the ONT's F/S/P, ONT-ID, existing service-port entries, and a free service-port ID are confirmed, continue with Case 1 (tagged) or Case 2 (untagged).

# Before you begin

Confirm the ONT is registered on the GPON interface and ONT ID shown in the example.

Confirm service-port ID is unused. Never reuse an ID that is already assigned.

Confirm VLAN 100 exists and is carried on the required uplink/trunk path.

Confirm the ONT's assigned line profile maps the selected GEM port to VLAN 100 as required.

Check existing services first. Do not remove or overwrite VLAN 2190 or any other working service unintentionally.

If an ONT already has an untagged service on the same GEM port, verify the design before adding another untagged service-port; traffic classification can conflict.

# Case 1 — Create a tagged VLAN 100 service-port

## Example parameters

| Service-port ID | 1941 |
| --- | --- |
| Service VLAN | 100 |
| GPON interface | 0/4/8 |
| ONT ID | 74 |
| GEM port | 2 |
| Customer-side VLAN | 100 (tagged) |
| Tag transformation | translate |

## Step 1 — Verify the registered ONT

Confirm ONT ID 74 is registered on GPON interface 0/4/8. Use the command syntax supported by your OLT release.

display ont info 0 4 8 74

## Step 2 — Inspect existing service-ports

Check the ONT's current service-port entries and confirm service-port ID 1941 is not already in use.

display service-port port 0/4/8 ont 74

## Step 3 — Check the line-profile GEM mapping

Find the ONT's assigned line-profile ID from the ONT information, then inspect that profile. Replace <LINE_PROFILE_ID> with the actual numeric ID. Confirm GEM 2 is appropriate for VLAN 100.

display ont-lineprofile gpon profile-id <LINE_PROFILE_ID>

## Step 4 — Verify VLAN 100

Confirm VLAN 100 exists and that the required uplink path carries it.

display vlan 100

## Step 5 — Create the tagged service-port

Run only after confirming the GEM mapping, VLAN path, and unused service-port ID.

service-port 1941 vlan 100 gpon 0/4/8 ont 74 gemport 2 multi-service user-vlan 100 tag-transform translate

## Step 6 — Verify the result

Confirm the service-port is present and its state is up. Test actual customer traffic as well; an up service-port alone does not prove end-to-end connectivity.

display service-port 1941
 display service-port port 0/4/8 ont 74

# Case 2 — Create an untagged VLAN 100 service-port

## Example parameters

| Service-port ID | 494 |
| --- | --- |
| Service VLAN | 100 |
| GPON interface | 0/2/2 |
| ONT ID | 56 |
| GEM port | 2 |
| Customer-side VLAN | Untagged |
| Tag transformation | default |

## Step 1 — Verify the registered ONT

Confirm ONT ID 56 is registered on GPON interface 0/2/2.

display ont info 0 2 2 56

## Step 2 — Inspect existing service-ports

Check current service-port entries and confirm service-port ID 494 is unused. Look for any existing untagged service on the same GEM port.

display service-port port 0/2/2 ont 56

## Step 3 — Check the line-profile GEM mapping

Find the assigned line-profile ID and confirm GEM 2 supports the intended VLAN 100 service. Check whether the line profile is shared with other ONTs before making any profile changes.

display ont-lineprofile gpon profile-id <LINE_PROFILE_ID>

## Step 4 — Verify VLAN 100

Confirm VLAN 100 exists and is carried on the required uplink path.

display vlan 100

## Step 5 — Create the untagged service-port

Run only after confirming the service-port ID, GEM mapping, and VLAN path.

service-port 494 vlan 100 gpon 0/2/2 ont 56 gemport 2 multi-service user-vlan untagged tag-transform default

## Step 6 — Verify the result

Confirm the service-port is present and up, then test customer connectivity.

display service-port 494
 display service-port port 0/2/2 ont 56

# Quick comparison

| Setting | Tagged service | Untagged service |
| --- | --- | --- |
| Service VLAN | vlan 100 | vlan 100 |
| Customer-side VLAN | user-vlan 100 | user-vlan untagged |
| Tag transformation | translate | default |
| Example service-port ID | 1941 | 494 |

# Troubleshooting notes

If the service-port command fails, inspect the exact CLI error and use the OLT's '?' help to confirm supported parameter order and options.

If the service-port remains down, verify ONT state, GEM mapping, VLAN configuration, uplink trunk membership, and physical/optical status.

If VLAN 100 is not mapped to the selected GEM port, a service-port command alone may not be sufficient.

Do not modify a line profile used by many ONTs without checking the impact.

Keep the existing VLAN 2190 service unless the change request explicitly calls for replacing it.

# Command summary

Tagged VLAN 100:

service-port 1941 vlan 100 gpon 0/4/8 ont 74 gemport 2 multi-service user-vlan 100 tag-transform translate

Untagged VLAN 100:

service-port 494 vlan 100 gpon 0/2/2 ont 56 gemport 2 multi-service user-vlan untagged tag-transform default

Safety note: This is a procedural reference, not a live configuration check. Verify every example against the target ONT, the current configuration, and the exact OLT software syntax before applying changes.