NEPAL AIRLINK

Huawei MA5683T Billing-System Integration

Technical Design: SSH/CLI Provisioning for Existing and New ONTs

Target platform: Huawei MA5683T · MA5600V800R015C00 · Patch SPC107

Purpose: Design a safe integration between Nepal Airlink billing, Huawei GPON OLTs, FreeRADIUS and MikroTik/BNG. This is a design guide; all CLI examples must be validated on the target OLT before production use.

# 1. Executive summary

Build a central provisioning service between billing and Huawei OLTs. Billing defines customer entitlement; provisioning maps it to approved templates, executes validated SSH commands, reads back configuration and records outcomes. FreeRADIUS and MikroTik/BNG continue to handle PPPoE authentication, accounting and speed policy according to the existing design.

Customer entitlement: package, speed and service status.

OLT configuration: DBA, T-CONT, GEM ports, line profile, ONT service profile, VLAN and service-port.

Internet policy: PPPoE account, speed policy, session control and accounting.

# 2. Recommended architecture

Billing/Admin Portal → Provisioning API and job queue → Profile catalogue and desired-state database → Restricted SSH worker → Huawei MA5683T OLTs. The provisioning service also coordinates FreeRADIUS and MikroTik/BNG policy updates through appropriate integration mechanisms.

| Component | Responsibility |
| --- | --- |
| Billing/Admin Portal | Customers, packages, ONT inventory and service orders. |
| Provisioning API/job queue | Validation, deduplication, locking, job state and audit. |
| Profile catalogue | Approved profile IDs, VLANs, GEM mappings and service templates. |
| SSH worker | Restricted OLT access, validated commands and read-back verification. |
| Huawei MA5683T | ONT registration, GPON profiles, GEM mapping and service-port forwarding. |
| FreeRADIUS | PPPoE authentication and accounting integration. |
| MikroTik/BNG | PPPoE termination and speed enforcement where configured. |

# 3. Design principles

Use reusable approved profiles instead of unique profiles per customer.

Keep billing package entitlement separate from optical transport configuration.

Do not edit shared profiles for one subscriber without checking all ONTs that use them.

Never delete/re-register an existing ONT just to change a package or add a VLAN.

Use a state machine: discovery, planning, validation, apply, verification and rollback.

Do not mark a service active until required OLT and PPPoE checks pass.

# 4. Profile and package catalogue

| Object | Example | Purpose |
| --- | --- | --- |
| DBA profile | DBA_FTTH_50M | Upstream bandwidth-allocation parameters. |
| Line profile | LINE_PPPoE_V100 | T-CONT/DBA bindings, GEM ports and mappings. |
| ONT service profile | SRV_ONT_4ETH | ONT-side port capabilities for compatible models. |
| Service VLAN | 100 | Service VLAN, subject to OLT/uplink configuration. |
| GEM mapping | GEM 1 → VLAN 100 | Maps service traffic to a GEM port. |
| Service-port | OLT-specific ID | Connects ONT/GEM service to OLT VLAN forwarding. |
| PPPoE policy | Home 50 Mbps | Authentication and speed policy in RADIUS/BNG. |

Example package mapping: Home 50 Mbps → approved DBA, line-profile and ONT service-profile templates, VLAN 100 and a PPPoE speed policy. Real numeric IDs must be verified per OLT. DBA primarily governs upstream allocation; it does not automatically create a symmetric 50 Mbps package.

# 5. Suggested database schema

## olts

id, name, management_ip, software_version, site_id, enabled, credential_ref

OLT inventory; credential_ref points to protected secrets, not plaintext passwords.

## olt_profiles

id, olt_id, profile_type, profile_id, profile_name, config_hash, verified_at

Verified DBA, line and service profiles scoped per OLT.

## service_templates

id, name, service_vlan, gem_index, user_vlan, tag_transform, line_profile_id, srv_profile_id, enabled

Approved mappings from service type to OLT configuration.

## ont_inventory

id, serial_number, olt_id, frame, slot, pon_port, ont_id, line_profile_id, srv_profile_id, status

ONT identity/location; enforce serial and location uniqueness.

## subscriber_services

id, customer_id, ont_inventory_id, package_id, service_template_id, pppoe_username, status, desired_config_version

Subscriber entitlement and target configuration.

## provisioning_jobs

id, subscriber_service_id, operation, idempotency_key, status, requested_by, started_at, completed_at, error_summary

Job lifecycle and duplicate-request prevention.

## provisioning_job_steps

id, job_id, step_name, command_template_id, result, verification_result, rollback_reference

Step execution and rollback evidence.

## ont_observed_state

ont_inventory_id, observed_at, observed_config, config_hash, drift_status

Last observed state for desired-versus-actual comparison.

# 6. SSH worker and job lifecycle

Accept the service order and validate customer, ONT, package, OLT and operation.

Acquire OLT-level and per-ONT locks to prevent conflicting jobs.

Connect with a restricted SSH account; verify host identity, prompt and software version.

Run read-only discovery and capture the before-state.

Build a change plan from approved templates; validate VLANs, profile IDs, GEM mappings and free IDs.

Apply only required commands. Avoid blind retries for resource-creation commands.

Read back the configuration and verify the resulting state.

Update RADIUS/BNG policy as appropriate and test PPPoE service.

Record the job outcome, sanitized audit trail and rollback reference.

# 7. Huawei CLI discovery commands

Run read-only discovery first. Command context and syntax can vary by release.

display version

display board

display vlan 100

display dba-profile all

display ont-lineprofile gpon all

display ont-srvprofile gpon all

display ont autofind all

display service-port all

For a particular ONT, inspect its status and profiles using syntax accepted by the target OLT. Use '?' when needed; do not assume IDs are identical across devices.

# 8. Illustrative DBA, line and service profile commands

Example DBA profile. Verify ID 13 is free and parameters suit the service:

config

dba-profile add profile-id 13 profile-name "FTTH_50M" type3 assure 1024 max 51200

display dba-profile profile-id 13

The values illustrate Type 3 with assure 1024 kbit/s and max 51200 kbit/s. This is upstream allocation, not a complete symmetric speed policy.

Illustrative line profile for VLAN 100:

ont-lineprofile gpon profile-id 40

tcont 1 dba-profile-id 13

gem add 1 eth tcont 1

mapping-mode vlan

gem mapping 1 0 vlan 100

commit

quit

This is a minimal example. Do not replace a production profile that carries other services (such as VLAN 2190) with it. Confirm mapping indexes and syntax on the target OLT.

Illustrative ONT service profile:

ont-srvprofile gpon profile-id 40

ont-port eth 4

commit

quit

Verify exact port syntax and ONT model capabilities before using.

# 9. New ONT registration and service-port

Illustrative registration command, only after confirming the serial is new and the ONT ID is free:

interface gpon 0/4

ont add 1 56 sn-auth "ONT_SERIAL_NUMBER" omci ont-lineprofile-id 40 ont-srvprofile-id 40

quit

Assumes interface gpon 0/4, port index 1 and ONT ID 56. Replace values with discovered/free values. Do not re-register an existing ONT just to change its package.

Illustrative service-port command:

service-port 1937 vlan 100 gpon 0/4/1 ont 56 gemport 1 multi-service user-vlan 100 tag-transform translate

Use only if service-port ID 1937 is free, VLAN 100 is configured, GEM 1 exists, and translate is intended. Verify syntax and tag behavior.

# 10. Existing ONT workflow

Find the ONT by serial number and confirm OLT/PON location.

Read current line/service profiles, DBA/T-CONT bindings, GEM mappings and service-ports.

Compare observed state with the requested change and list services that must remain unchanged.

For an existing VLAN 2190 service, preserve its GEM mapping and service-port while adding VLAN 100 if requested.

Apply the smallest change, verify it, then update the PPPoE policy if needed.

# 11. New ONT workflow

Discover ONT serial and physical PON port.

Validate ONT model and approved line/service templates.

Check VLAN availability and find free ONT ID and service-port ID.

Create missing reusable profiles only after checking for equivalent profiles and conflicts.

Register the ONT and create the intended service-port.

Verify ONT online state, optical status, GEM/VLAN mapping and service-port.

Configure/activate PPPoE in FreeRADIUS and apply the intended MikroTik/BNG policy.

Mark service active only after required checks pass.

# 12. Verification checklist

| Check | Expected result |
| --- | --- |
| ONT registration | Correct ONT is present and online. |
| Profiles | Expected line-profile and service-profile IDs are applied. |
| Optical status | Link and optical levels are within accepted limits. |
| GEM mapping | Required VLAN maps to intended GEM port. |
| Service-port | Correct ONT, GEM, service VLAN and user VLAN/tag transform. |
| VLAN/uplink | VLAN is available and carried upstream. |
| PPPoE | Credentials authenticate and session is established. |
| Speed policy | Package speed is enforced at the configured enforcement point. |

# 13. Rollback and failure handling

| Failure | Response |
| --- | --- |
| SSH timeout before change | Reconnect and inspect state before retrying. |
| Timeout after ONT-add | Check whether registration succeeded before retrying. |
| Service-port failure | Inspect VLAN, GEM, ONT and ID allocation. |
| Verification failure | Stop activation and restore only changes made by the failed job, where safe. |
| Existing service affected | Stop automation, preserve evidence and escalate/restore known-good state. |
| Shared profile in use | Do not delete it; use an approved migration plan. |

Rollback procedure:

Capture before-state and identify every resource created or modified by the job.

Remove a new service-port only if it belongs to the failed job and no other service depends on it.

Restore modified configuration only after checking current state and dependencies.

Re-verify the failed service and any existing services potentially affected.

Record ROLLED_BACK, FAILED_NEEDS_REVIEW or PARTIALLY_APPLIED with an audit trail.

# 14. Security and operational safeguards

Use SSH host-key verification, restricted credentials and firewall rules for approved management IPs.

Never store plaintext passwords in the OLT inventory or logs; use a secrets manager/reference.

Only approved command templates may reach the SSH worker; never accept raw CLI from a billing form.

Use idempotency keys and per-OLT/per-ONT locks to prevent duplicate/conflicting jobs.

Log operator, service order, before-state, steps, verification and rollback reference; sanitize secrets.

Require approval for destructive operations and shared-profile migrations.

Maintain configuration backups and test rollback on a non-production ONT.

Use a defined save/persistence policy; do not blindly issue save after every job.

# 15. Implementation roadmap

## Phase 1 — Read-only discovery

Inventory profiles, VLANs, ONTs and service-ports on one test OLT.

## Phase 2 — Catalogue and mapping

Create approved templates and map packages to OLT and RADIUS/BNG policies.

## Phase 3 — Controlled provisioning

Test one new ONT and one existing-ONT VLAN addition with operator approval.

## Phase 4 — Failure testing

Test timeouts, duplicate requests, invalid VLANs, profile conflicts and rollback.

## Phase 5 — Gradual rollout

Enable more OLTs gradually and monitor success rate, service impact and job duration.

# 16. First OLT discovery checklist

Collect these outputs from a representative OLT before finalizing production templates:

display version

display dba-profile all

display ont-lineprofile gpon all

display ont-srvprofile gpon all

display vlan all

display service-port all

Also capture one working ONT using VLAN 2190, including its line profile, service profile, GEM mappings and service-port entries. Redact serial numbers, credentials and public IPs as appropriate.

# 17. Limitations and production notes

Numeric IDs and CLI examples are illustrative, not validated commands for every MA5683T.

Syntax, mapping indexes, supported options and persistence behavior can vary by release and ONT model.

DBA governs upstream allocation; symmetric speeds may require downstream and BNG/MikroTik policies.

A successful CLI response does not prove end-to-end Internet connectivity.