HUAWEI MA5683T

GPON / Management / Service Provisioning

FIELD CONFIGURATION MANUAL

| Item | Value |
| --- | --- |
| Product | Huawei MA5683T |
| VRP / Software | MA5600V800R015C00 |
| Patch | SPC107 |
| Primary use | GPON customer provisioning, VLAN/service-port management, OLT administration |
| Example OLT name | Bijule-Olt-hauwei |
| Manual purpose | Fast, repeatable field configuration and troubleshooting |

⚠ PRODUCTION SAFETY This is a field runbook, not a substitute for checking the live configuration. Before changing a VLAN, service-port, line profile, service profile, or ONT binding, capture the current configuration and verify the exact port/ONT/service-port IDs. Never delete a working service-port until the replacement has been verified.

Prepared from the Bijule-Olt configuration history and cross-checked against Huawei MA5600T/MA5683T documentation.

# Contents

Tip: In Microsoft Word, right-click the table of contents and choose “Update Field” after editing.

# 1. Quick Reference — Daily Commands

| Task | Command |
| --- | --- |
| Enter configuration mode | enable → config |
| Show software / patch | display version |
| Find ONTs automatically | display ont autofind all |
| Show ONT status by SN | display ont info by-sn <SN> |
| Show ONTs on a PON port | display ont info <port-id> all |
| Find next free service-port | display service-port next-free-index |
| Show service-port | display service-port <ID> |
| Show VLAN | display vlan <VLAN-ID> |
| Show IP interfaces | display ip interface brief |
| Show current configuration | display current-configuration |
| Show only VLAN-related config | display current-configuration ¦ include "vlan" |
| Show only IP-related config | display current-configuration ¦ include "ip" |
| Show terminal users | display terminal user |
| Show SSH users | display current-configuration ¦ include "ssh user" |
| Save configuration | save |

COMMAND CONTEXT MATTERS Huawei uses different prompts for global configuration, GPON interface, line-profile, and service-profile views. A command that works under interface gpon 0/4 may not work from config mode. If unsure, use “?” at the current prompt before changing anything.

# 2. Management VLAN, Management IP & Gateway

## 2.1 Target design

| Parameter | Example |
| --- | --- |
| Management VLAN | 199 |
| OLT management IP | 172.16.16.4/24 |
| Management subnet | 172.16.16.0/24 |
| Gateway | 172.16.16.1 |
| Existing VLAN 199 on Bijule OLT | Already present |
| Existing IP seen in history | 172.16.16.3/24 — verify before adding .4 |

IMPORTANT FOR YOUR BIJULE OLT Your captured configuration already contains “vlan 199 smart” and an existing “ip address 172.16.16.3 255.255.255.0”. Do not blindly add 172.16.16.4 until you identify where .3 is configured and confirm that .4 is unused.

## 2.2 Pre-check

display current-configuration | include "172.16.16.3"
display current-configuration | include "interface vlanif"
display vlan 199
display port vlan all
display ip interface brief

## 2.3 Create VLAN 199 if it does not already exist

system-view
vlan 199 smart
quit

For the Bijule OLT shown in the history, VLAN 199 already exists, so skip this step.

## 2.4 Add VLAN 199 to the management/uplink port

system-view
port vlan 199 0/9 1
quit

Example based on the captured configuration format. Replace 0/9 1 with the actual uplink port and verify its existing VLAN membership first. Do not overwrite a live trunk/service configuration.

## 2.5 Create the VLANIF management interface

system-view
interface vlanif 199
ip address 172.16.16.4 255.255.255.0
quit

VERIFY BEFORE USING THIS IP If the OLT already owns 172.16.16.3/24 on another management interface, keep the existing interface intact. The goal is to add .4 only where the architecture permits it; never create duplicate/overlapping interface addresses accidentally.

## 2.6 Add the default/static route

system-view
ip route-static 0.0.0.0 0.0.0.0 172.16.16.1
quit

Verification

display ip routing-table
ping 172.16.16.1
ping 172.16.16.4

## 2.7 Management checklist

VLAN 199 exists and is the intended management VLAN.

VLAN 199 is permitted on the correct uplink/trunk.

172.16.16.4 is unused and not duplicated elsewhere.

Gateway 172.16.16.1 responds.

SSH/Telnet access is reachable from the management network.

Only after testing: save the configuration.

# 3. VLAN Creation & Pass-Through / Trunk

## 3.1 Create a service VLAN

system-view
vlan 100 smart
quit

## 3.2 Pass VLAN through an uplink

system-view
port vlan 100 0/9 1
quit

The exact uplink port syntax depends on the existing port role and configuration. Always inspect the uplink first.

Pre/post-check

display port vlan all
display current-configuration
display vlan 100

## 3.3 Example — management VLAN 199

vlan 199 smart
port vlan 199 0/9 1
interface vlanif 199
ip address 172.16.16.4 255.255.255.0
quit
ip route-static 0.0.0.0 0.0.0.0 172.16.16.1

TRUNK SAFETY RULE If the uplink already carries customer VLANs, add the new VLAN to the existing trunk; do not replace the trunk's entire VLAN list unless you have a complete backup and a maintenance window.

# 4. OLT User, Administrator Role, SSH & Telnet

## 4.1 Existing user model on this platform

Your MA5683T configuration history shows the legacy MA5600T terminal-user model, for example users such as root, portal and chatra, plus SSH authentication entries. Huawei documentation for this platform describes terminal-user creation as an interactive “terminal user name” operation, followed by user profile, level, re-enter count and appended information.

YOUR SPC107 CLI BEHAVIOR During the Bijule-Olt session, “terminal user ?” showed only “apdinfo” and “password”. Therefore, if “terminal user name” is not exposed/accepted at your current privilege/context, do NOT invent a one-line encrypted-password command. Use the exact CLI help available to the privileged account or the OLT's supported LCT/NMS user-management workflow.

## 4.2 Standard MA5600T/MA5683T user-creation flow

system-view
terminal user name
User Name(length<6,15>): shekhar
User Password(length<6,15>): <your-password>
Confirm Password(length<6,15>): <your-password>
User profile name(<=15 chars)[root]: <Enter>
User's Level:
1. Common User
2. Operator
3. Administrator: 3
Permitted Reenter Number(0--4): 4
User's Appended Info(<=30 chars): admin
Adding user succeeds
Repeat this operation? (y/n)[n]: n

Huawei's MA5600T documentation shows this interactive flow. If your live SPC107 CLI does not expose the “name” subcommand, first resolve the privilege/context issue rather than copying encrypted password strings from the current configuration.

## 4.3 Verify the new user

display terminal user
display terminal user name shekhar

## 4.4 Change an existing user's password

terminal user password
User Name(<=15 chars): shekhar
New Password(length<6,15>): <new-password>
Confirm Password(length<6,15>): <new-password>
Information takes effect
Repeat this operation? (y/n)[n]: n

## 4.5 Change administrator level

terminal user level
User Name(<=15 chars): shekhar
1. Common User
2. Operator
3. Administrator
User's Level: 3
Confirm Level: 3
Repeat this operation? (y/n)[n]: n

## 4.6 Configure SSH user authentication

ssh user shekhar authentication-type password

Verification

display current-configuration | include "ssh user"
display ssh server status

## 4.7 Telnet

display current-configuration | include "telnet"
display telnet server status

If Telnet is required on your firmware, enable it using the command exposed by “display telnet server status” / “telnet ?”. Prefer SSH for routine administration because Telnet is plaintext.

DO NOT COPY ENCRYPTED PASSWORDS Your captured configuration contains password hashes for existing accounts. This manual intentionally does not reproduce them. Create a fresh password for shekhar and test it before removing or disabling any existing administrator.

# 5. DBA, Line Profile & Service Profile

## 5.1 Understand the three building blocks

| Object | Purpose | Typical checks |
| --- | --- | --- |
| DBA profile | Bandwidth allocation parameters used by T-CONTs. | display dba-profile |
| ONT line profile | T-CONT ↔ DBA, GEM ports and VLAN mapping. | display ont-lineprofile gpon all |
| ONT service profile | ONT port capability and Ethernet/POTS/VLAN behavior. | display ont-srvprofile gpon all |

## 5.2 Check an existing line profile before changing it

display ont-lineprofile gpon profile-id 4
display ont-lineprofile gpon profile-id 4

Use the exact profile ID used by the target ONT. Your history shows many ONTs using line-profile-id 4.

## 5.3 Example line profile structure

ont-lineprofile gpon profile-id 4
tcont 1 dba-profile-id 4
gem add 2 eth tcont 1
gem mapping 2 0 vlan 100
commit
quit

Example only. Do not paste this into a production profile until you have inspected the existing profile. Your captured configuration already shows GEM 2 mappings for VLANs 2208, 2050 and 100, so changing GEM mappings can affect existing customers.

## 5.4 Check service profile

display ont-srvprofile gpon all
display ont-srvprofile gpon profile-id 4

## 5.5 Example service profile

ont-srvprofile gpon profile-id 4
ont-port eth 4
port vlan eth 1 translation 100 user-vlan 100
commit
quit

Use the actual ONT hardware port count and existing service-profile design. The service profile's port capability must match the physical ONT capability.

## 5.6 Re-bind an ONT to a different line/service profile

system-view
interface gpon 0/4
ont modify <port-id> <ont-id> ont-lineprofile-id <new-line-profile-id>
ont modify <port-id> <ont-id> ont-srvprofile-id <new-service-profile-id>
quit

PROFILE CHANGE WARNING Changing a line/service profile can interrupt the customer. Before changing it, record the ONT's current profiles, service-port(s), GEM mapping and VLAN. Huawei guidance also notes that profile compatibility/match state matters.

# 6. ONT / ONU Registration by Serial Number

## 6.1 Find an automatically discovered ONT

display ont autofind all

For a specific PON interface, enter the GPON interface first and use the port-specific autofind command if required by the CLI.

## 6.2 Find a free ONT ID

display ont info 0 4 7 all

Your field notation: 0 = chassis/frame, 4 = slot, 7 = GPON port. Confirm the exact F/S/P shown by the OLT before using an ONT ID.

## 6.3 Register an ONT using SN authentication

system-view
interface gpon 0/4
ont add 7 23 sn-auth "48575443XXXXXXXX" omci ont-lineprofile-id 4 ont-srvprofile-id 4 desc "Customer-XXX"
quit

In this example, GPON port 7 and ONT-ID 23 are used. The exact ONT-add syntax is context-sensitive: Huawei's MA5600T V800R015 documentation uses the GPON interface context and `ont add <port-id> <ont-id> ...`.

## 6.4 Example from the Bijule history

interface gpon 0/4
ont add 7 118 sn-auth "485754439785B59A" omci ont-lineprofile-id 4 ont-srvprofile-id 4 desc "buiipafirstcutomer"
quit

Your historical command appeared as `ont add 118 0 ...` in one context and as `ont add 2 23 ...` in another. Treat the port/ONT-ID order as context-specific and verify with `ont ?` at the current GPON prompt before production use.

## 6.5 Confirm automatically discovered ONTs in bulk

interface gpon 0/4
ont confirm 7 all sn-auth omci ont-lineprofile-id 4 ont-srvprofile-id 4
quit

Bulk confirmation is appropriate when the ONTs discovered on the same PON should receive the same authentication/profile treatment. Review the autofind list first; do not bulk-confirm unknown customer devices on a live PON.

## 6.6 Check ONT status

display ont info by-sn 485754439785B59A

Specific ONT

display ont info 7 23

All ONTs on PON port 7

display ont info 7 all

| State | Meaning / first action |
| --- | --- |
| Control flag: active | ONT is activated. |
| Run state: online | Optical/registration state is online. |
| Config state: normal | Configuration applied successfully. |
| Match state: match | Bound capability/profile matches ONT capability. |
| Run state: offline | Check fiber, optics, splitter, ONT power and alarms. |
| Config state: failed | Use failure diagnostics and inspect profile compatibility. |
| Match state: mismatch | Check ONT capability and line/service profile. |

# 7. Move an Existing ONT/Customer to a New VLAN

## 7.1 Change the service-port VLAN — preferred method

If the ONT and profiles stay the same but the customer's service VLAN changes, the usual change is the service-port. This avoids re-registering the ONT SN unnecessarily.

system-view
undo service-port 32
service-port 32 vlan 100 gpon 0/4/2 ont 6 gemport 2 multi-service user-vlan 100 tag-transform translate
quit

## 7.2 Create a new service-port instead of reusing an index

display service-port next-free-index
system-view
service-port 969 vlan 100 gpon 0/4/1 ont 56 gemport 2 multi-service user-vlan 100 tag-transform translate
quit

INDEX SAFETY Before creating a service-port, check the next-free index. If you intentionally replace an existing service-port, record the old command first. A wrong ONT ID, GEM port or VLAN can break a customer's service immediately.

## 7.3 Verify service-port

display service-port 32
display service-port 969
display service-port port 0/4/2 ont 6
display service-port port 0/4/1 ont 56

## 7.4 Move an ONT from VLAN A to VLAN B — example

# Existing:
service-port 32 vlan 2050 gpon 0/4/2 ont 6 gemport 2 multi-service user-vlan 2050 tag-transform translate

# Change to VLAN 100:
undo service-port 32
service-port 32 vlan 100 gpon 0/4/2 ont 6 gemport 2 multi-service user-vlan 100 tag-transform translate

# 8. Individual & Bulk Customer Migration Workflow

## 8.1 Individual customer

Find ONT by SN: `display ont info by-sn <SN>`.

Record F/S/P, ONT-ID, line profile, service profile, GEM port and current service-port.

Check the target VLAN exists: `display vlan <new-vlan>`.

Check the uplink/trunk carries the target VLAN.

Change or create the service-port with the target VLAN.

Verify ONT status and service-port.

Test customer traffic.

Save only after verification.

## 8.2 Bulk customer migration

# Discovery / audit
display ont info by-sn <SN>
display service-port port <F/S/P> ont <ONT-ID>

# For each service-port, record:
# old service-port ID | old VLAN | F/S/P | ONT-ID | GEM | new VLAN

# Example replacement
undo service-port <old-id>
service-port <old-id> vlan <new-vlan> gpon <F/S/P> ont <ont-id> gemport <gem> multi-service user-vlan <new-vlan> tag-transform translate

BULK CHANGE METHOD For dozens/hundreds of customers, prepare a change sheet first and verify every F/S/P + ONT-ID + GEM mapping. Do not run a blind bulk delete/recreate on a production OLT.

# 9. Change / Troubleshoot Line & Service Profiles

## 9.1 Line profile checklist

Check whether the profile is bound to customers.

Check DBA/T-CONT assignments.

Check GEM port definitions.

Check GEM-to-VLAN mappings.

Commit profile changes.

Verify affected ONTs' match/config state.

If possible, test with one customer before rolling out.

display ont-lineprofile gpon all
display ont-lineprofile gpon profile-id 4
display ont info 7 23
display ont capability 7 23

## 9.2 Service profile checklist

display ont-srvprofile gpon all
display ont-srvprofile gpon profile-id 4
display ont info 7 23

## 9.3 Common profile symptoms

| Symptom | Likely area | Check |
| --- | --- | --- |
| ONT online but no traffic | Service-port / VLAN / GEM mapping | display service-port; line profile GEM mapping |
| Match mismatch | Service profile/capability mismatch | display ont capability; service profile |
| Config failed | Profile command/capability issue | display ont failed-configuration |
| Online but wrong VLAN | Service-port or GEM mapping | display service-port; gem mapping |
| All customers on same profile affected | Shared profile change | Inspect line/service profile before modifying |

# 10. Status & Troubleshooting Command Library

## 10.1 ONT status

display ont info by-sn <SN>
display ont info <port-id> <ont-id>
display ont info <port-id> all
display ont autofind all
display ont capability <port-id> <ont-id>

## 10.2 Optical / PON checks

display port info <port-id>
display ont optical-info <port-id> <ont-id>
display ont optical-info by-sn <SN>

If a command is unavailable on this patch level, use `?` at the GPON prompt and use the supported equivalent.

## 10.3 Service-port / VLAN checks

display service-port next-free-index
display service-port <service-port-id>
display service-port port <F/S/P> ont <ONT-ID>
display vlan <vlan-id>
display vlan all
display port vlan all

## 10.4 System / board checks

display version
display board 0
display board
display fan
display power
display alarm active
display alarm history

## 10.5 Management / routing

display ip interface brief
display ip routing-table
display current-configuration | include "ip"
display current-configuration | include "vlan"
ping <gateway>
ping <management-host>

## 10.6 User / session checks

display terminal user
display client
display current-configuration | include "ssh user"
display ssh server status

# 11. Fast Troubleshooting Matrix

| Problem | Check first | Then check | Typical fix |
| --- | --- | --- | --- |
| ONT offline | display ont info | optical info, fiber/ONT power, alarms | Repair optical path / ONT; verify activation |
| ONT online, no Internet | display service-port | VLAN, GEM mapping, uplink | Correct service-port/VLAN/GEM |
| Match mismatch | display ont capability | service profile / line profile | Bind compatible profile |
| Config failed | display ont failed-configuration | profile capability | Correct profile/config item |
| New VLAN not working | display vlan <VLAN>; display port vlan all | uplink/trunk | Permit VLAN on uplink |
| Can't reach management IP | display ip interface brief | route + VLAN trunk + gateway | Correct VLANIF/uplink/route |
| SSH fails, Telnet works | display ssh server status | ssh user config | Enable/configure SSH and authentication |
| Service-port ID conflict | display service-port next-free-index | display service-port <ID> | Use next free ID or deliberately replace existing |
| Wrong customer VLAN | display service-port port ... ont ... | line profile GEM mapping | Correct service-port or GEM mapping |

# 12. Production Change Workflow

Identify the customer/ONT by SN and exact F/S/P + ONT-ID.

Capture the existing service-port and profile configuration.

Check target VLAN exists and is carried on the uplink.

Check for free service-port index if creating a new service-port.

Make the smallest possible change.

Verify ONT state: active / online / normal / match.

Verify service-port and VLAN.

Test customer traffic.

Only then save the configuration.

Record the change in the NOC/service record.

# Final verification
display ont info <port-id> <ont-id>
display service-port port <F/S/P> ont <ONT-ID>
display vlan <vlan-id>
display current-configuration | include "<unique-customer-keyword>"

# Save after verification
save

# 13. Backup & Rollback Discipline

Before a major change, capture `display current-configuration` or the relevant configuration section.

For service changes, save the old service-port command in your ticket/change record.

For profile changes, record the complete profile before editing.

Do not delete a profile that is still bound to customers.

After a successful change, save the configuration according to your NOC change policy.

Keep a dated configuration backup outside the OLT.

ROLLBACK PRINCIPLE For a service-port migration, rollback is normally to restore the exact previous service-port command. For a profile change, rollback should restore the previous profile contents or re-bind the ONT to the previous known-good profile.

# 14. Copy/Paste Templates

## 14.1 New ONT

system-view
interface gpon <frame>/<slot>
ont add <pon-port> <ont-id> sn-auth "<SN>" omci ont-lineprofile-id <line-id> ont-srvprofile-id <srv-id> desc "<customer>"
quit
display ont info <pon-port> <ont-id>

## 14.2 New customer service-port

display service-port next-free-index
system-view
service-port <new-id> vlan <vlan> gpon <F/S/P> ont <ont-id> gemport <gem> multi-service user-vlan <vlan> tag-transform translate
quit
display service-port <new-id>

## 14.3 Change existing customer VLAN

system-view
undo service-port <service-port-id>
service-port <service-port-id> vlan <new-vlan> gpon <F/S/P> ont <ont-id> gemport <gem> multi-service user-vlan <new-vlan> tag-transform translate
quit
display service-port <service-port-id>
display ont info <pon-port> <ont-id>

## 14.4 Management VLAN

system-view
vlan 199 smart
port vlan 199 <uplink-frame>/<uplink-slot> <uplink-port>
interface vlanif 199
ip address 172.16.16.4 255.255.255.0
quit
ip route-static 0.0.0.0 0.0.0.0 172.16.16.1
quit
display ip interface brief
display ip routing-table

# 15. References & Notes

This manual was assembled from the Bijule-Olt-hauwei command/configuration history supplied for this project and cross-checked against Huawei MA5600T/MA5603T/MA5608T commissioning/configuration material covering V800R015C00 and related MA5600T generations.

Huawei SmartAX MA5600T/MA5603T/MA5608T V800R015C00 Commissioning and Configuration Guide — GPON/FTTH procedures, ONT confirmation, bulk confirmation and status checks.

Huawei MA5600T/MA5603T user-management documentation — terminal user creation, password, level, user-profile, re-enter and appended-information operations.

Huawei support troubleshooting guidance — ONT profile mismatch, capability checks and profile correction.

VERSION-SPECIFIC NOTE The manual is targeted at MA5683T / MA5600V800R015C00 / SPC107. Huawei CLI behavior can vary with board type, patch level, operating mode and privilege. When a command is not accepted, use `?` at the exact CLI prompt and capture the output before changing the configuration.

# Appendix A — Bijule OLT Known Values

| Known item | Value from project history |
| --- | --- |
| OLT hostname | Bijule-Olt-hauwei |
| Product | MA5683T |
| VRP | MA5600V800R015C00 |
| Patch | SPC107 |
| Existing management/service VLAN | 199 exists as smart VLAN |
| Known management IP seen | 172.16.16.3/24 |
| Planned management IP | 172.16.16.4/24 |
| Planned gateway | 172.16.16.1 |
| Known uplink descriptions | 0/9/0 = to-regmitar; 0/9/1 = from-Bijuel |
| Common line profile in history | 4 |
| Example GEM port in history | 2 |
| Known GEM mappings | GEM 2 → VLAN 2208, 2050, 100 |

# Appendix B — Field Notes

Do not assume an ONT ID is free from memory; inspect the actual PON port.

Do not assume a service-port ID is free; use `display service-port next-free-index`.

Do not change a shared line/service profile casually; one profile may serve many customers.

When moving a customer to another VLAN, normally change the service-port rather than re-registering the ONT.

Keep SN, F/S/P, ONT-ID, line profile, service profile, GEM and service-port together in the customer record.

Prefer SSH over Telnet for management whenever possible.

MA5683T FIELD MANUAL

“Check first. Change second. Verify third. Save last.”